Brief User Manual for MUNIU Radar ROS2 Driver

| Project Name | ROS2_MUNIU_RADAR |
| --- | --- |
| Document No. |  |
| Version | V1.0.1 |
| Security Level |  |
| Author | Diao Tianping |
| Reviewer | Diao Tianping |
| Approver |  |

All Rights Reserved Beijing MUNIU Leadership Technology Co., Ltd.

This document and all its contents are the property of Beijing MUNIU Leadership Technology Co., Ltd. (MUNIU Technology) and are protected by Chinese copyright laws and applicable international copyright conventions. Without the prior written authorization of MUNIU Technology, no one may copy, distribute, disseminate, modify or use part or all of the contents of this document in any form. Violators will be held legally responsible.

。

Document Update History

| Date | Updater | Version | Description |
| --- | --- | --- | --- |
| 2026.01.14 | DTP | V1.0.1 | Initial version completed |

Table of Contents

Brief User Manual for MUNIU Radar ROS2 Driver	1

1、Overview	4

2、Environment Preparation	4

1). System Dependencies	4

2).Dependency Installation Commands	4

3). Workspace Preparation	5

3、Configuration File Modification	5

1) . CAN Protocol Radar (K77 as an Example)	5

2). UDP Protocol Radar (I79 as an Example)	6

4、Node Operation	7

1). Start Radar Node	7

2). Stop Radar Node	7

5、Data Verification	8

1). Topic Subscription Test Program	8

2). View Published Topic Data	8

3).View 3D Visualization	9

4. Topic Data Format	10

5. CAN Device Debugging (For CAN Protocol Radar)	14

6、Common Troubleshooting	15

7、Notes	16

1、Overview

Radar_muniu is a ROS2-based software library and toolset that enables radar sensors (via CAN bus) to stream object/target data into a 3D visualization environment for analysis, debugging, and integration into robotics or vehicle systems.

It currently supports three MUNIU radar models: I79, K77, and T79.

2、Environment Preparation

1). System Dependencies

·  Operating System: Ubuntu 24.04 LTS

·  ROS2 Version: Jazzy Jalisco

·  Basic Dependencies: ROS2 Jazzy Desktop Edition, CAN-related tools, PCAN driver, YAML parsing library, network tools

2).Dependency Installation Commands

Install ROS2 Jazzy basic environment

- sudo apt update && sudo apt install ros-jazzy-desktop

·  Install CAN-related dependencies (for CAN protocol radar)

- sudo apt install ros-jazzy-can-msgs ros-jazzy-socketcan-bridge

·  Install YAML parsing and network-related dependencies

- sudo apt install python3-yaml net-tools

·  Install Boost library

sudo apt install -y libboost-all-dev

·  Install Python and C++ environment

sudo apt install -y build-essential g++ python3-dev

3). Workspace Preparation

· Create and initialize ROS2 workspace

- mkdir -p ~/radar_ws & cd ~/radar_ws

· Extract the radar_ros2_pkg.tar.bz2 compressed package

- tar -xjvf radar_ros2_pkg.tar.bz2

- cd ~/radar_ws/radar_ros2_pkg

3、Configuration File Modification

Configuration file path: install/radar_muniu/share/radar_muniu/config/radar_muniu_config.yaml. Modify the configuration according to radar hardware parameters:

- . CAN Protocol Radar (K77 as an Example)

## 2). UDP Protocol Radar (I79 as an Example)

4、Node Operation

1). Start Radar Node

Start the radar node and RViz2 program via script

- cd ~/radar_ws

- ./publish.sh

Note: publish.sh is started through the radar_launch.py file under the path install/radar_muniu/share/radar_muniu/launch

- . Stop Radar Node

Use Ctrl + C to stop the node from running.

5、Data Verification

1). Topic Subscription Test Program

cd ~/radar_ws & ./subscriber.sh

I79 Diagram

2). View Published Topic Data

ros2 topic list

The corresponding outputs for the three models are as follows:

K77:

/radar_muniu_k77/data/object_list（Target data topic）

/radar_muniu_k77/vis/object_list (RViz2 target topic)

I79:

/radar_muniu_i79/data/detection_list（Point cloud data topic）

/radar_muniu_i79/data/object_list(Target data topic)

/radar_muniu_i79/data/version_info (Version data topic)

/radar_muniu_i79/vis/object_list (RViz2 target topic)

/radar_muniu_i79/vis/detection_list(RViz2 point cloud topic)

T79：

/radar_muniu_t79/data/object_list(Target data topic)

/radar_muniu_t79/vis/t79_list(RViz2 point cloud topic)

3).View 3D Visualization

The local coordinate system IDs for the three radar models K77, I79, and T79 correspond to muniu_k77_link, muniu_i79_link, and muniu_t79_link respectively, which are generated according to the YAML file configuration.

4. Topic Data Format

1) I79

# detection

uint16 measurement_id

float32 azimuth_angle

float32 azimuth_angle_std

uint8 invalid_flags

float32 elevation_angle

float32 elevation_angle_std

float32 range

float32 range_std

float32 range_rate

float32 range_rate_std

int8 rcs

int8 snr

uint8 positive_predictive_value

uint8 classification

uint8 ambiguity_flag

#objecgt

uint32 object_id

uint16 object_age

uint8 object_status_measurement

uint8 object_status_movement

uint8 position_reference

float32 position_x

float32 position_x_std

float32 position_y

float32 position_y_std

float32 position_z

float32 position_z_std

float32 position_orientation

float32 position_orientation_std

float32 dynamics_vel_x

float32 dynamics_vel_x_std

float32 dynamics_vel_y

float32 dynamics_vel_y_std

float32 dynamics_vel_z

float32 dynamics_vel_z_std

float32 dynamics_acce_x

float32 dynamics_acce_x_std

float32 dynamics_acce_y

float32 dynamics_acce_y_std

float32 dynamics_acce_z

float32 dynamics_acce_z_std

float32 dynamics_orientation_rate

float32 dynamics_orientation_rate_std

float32 existence_prob

float32 obstacle_prob

uint8 classification

uint8 classification_prob

float32 shape_length_mean

float32 shape_length_std

float32 shape_width_mean

float32 shape_width_std

float32 shape_height_mean

float32 shape_height_std

#version

std_msgs/Header header

uint64 crc

uint32 length

uint32 sqc

uint32 data_id

uint32 frame_count

uint8[3] protocol_version

uint8[4] hw_pcb_ver

uint8[4] hw_bom_ver

uint8[4] cfg_version

uint8[4] sw_pl_ver

uint8[4] sw_ps_ver

uint8[3] alg_pt_ver

uint8[3] alg_trk_ver

uint8[3] calib_ver

uint8[16] sn_shell

2)K77 &T79

#DBC Message

std_msgs/Header header

uint32 id

string name

radar_msgs/DbcSignal[] signals

# DBC Signal

string  name

float64 value

string  unit

5. CAN Device Debugging (For CAN Protocol Radar)

1）Check CAN device status

ip link show can0 # If using SocketCAN

pcanview # If using PCAN device

- Monitor CAN bus data (verify if the radar sends messages)

candump can0 # SocketCAN

pcanfdump -f 500k -d 2000k /dev/pcan32 # PCAN (channel number must match)

6、Common Troubleshooting

| Fault Phenomenon | Possible Causes | Solutions |
| --- | --- | --- |
| CAN device not recognized | PCAN driver not installed successfully; Mismatched channel number/baud rate; 3. Insufficient device permissions | 1. Execute ls /dev/pcan* to verify the driver and reinstall if necessary; 2. Check that the configuration file matches the hardware parameters; 3. Execute sudo chmod 777 /dev/pcan32 (adjust the path according to the actual situation) |
| No data from UDP radar | Local IP/port not connected to the radar's IP/port; Radar not configured with data transmission target; 3. Firewall blocking | 1. Test connectivity with ping radar_IP; 2. Check radar network configuration; 3. Temporarily disable the firewall with sudo ufw disable for testing |
| No data output from topics | Verify the message IDs and signal names in parse_rule; Validate the DBC file path and integrity; 3. Check if the install_* parameters are reasonable | 1. Verify the message IDs and signal names in parse_rule; 2. Validate the DBC file path and integrity; 3. Check if the install_* parameters are reasonable |

7、Notes

· All configuration parameters (such as CAN baud rate, UDP port, DBC signal names) must be consistent with the radar hardware manual; otherwise, data parsing will fail.

·  Before running the node, ensure the radar hardware is powered on and connected properly: the CAN bus termination resistor is normal, and the UDP radar is configured with a static IP and in the same network segment as the local IP.

·  If ROS2 dependencies are missing under Ubuntu 24.04, they can be installed automatically with the following command:

rosdep install --from-paths src --ignore-src -r -y